MQTT Broker Load Testing Report

1.  Objective

The objective of this experiment is to evaluate the performance and scalability of an MQTT broker hosted on Azure by simulating multiple IoT devices. The test analyzes how the broker behaves under increasing load conditions.

2.  Tools & Technologies Used

- Python (paho-mqtt)

- MQTT Protocol (Secure - TLS)

- Azure Virtual Machine (MQTT Broker)

- Azure Metrics (Monitoring)

- Visual Studio Code

3.  Test Setup

- Broker Endpoint: mqtts://mqtt.protonest.co

- Port: 8883

- Authentication: Username & Password (secure, not hardcoded)

- Protocol: MQTT over TLS

Each simulated device:

- Uses a unique username and client ID

- Sends JSON-formatted sensor data

- Connects, sends data, disconnects, and reconnects

4.  Testing Methodology

The test was conducted in multiple stages by gradually increasing the number of simulated devices:

| Stage | Number of Devices |
| --- | --- |
| 1 | 100 |
| 2 | 200 |
| 3 | 300 |
| 4 | 400 |
| 5 | 500 |

For each stage:

- All devices connected simultaneously

- All devices sent data simultaneously

- All devices disconnected

- The process was repeated in cycles

5.  Metrics Monitored

The following metrics were observed using Azure Monitor:

- CPU Usage (%)

- Available Memory (%)

- Network In Total

- Network Out Total

6.  Observations (Add screenshots here)

Insert screenshots for each stage:

- 100 devices

- 200 devices

- 300 devices

- 400 devices

- 500 devices

7.  Results & Analysis

CPU Usage

- CPU usage remained very low (below ~1%)

- Indicates that the broker is not CPU-bound

Memory Usage

- Memory remained stable around ~30%

- No significant memory pressure observed

Network Activity

- Network In and Network Out showed clear spikes during each test cycle

- The spikes correspond to:

  - Device connections

  - Data transmission

  - Broker responses

System Behavior

- The broker handled all device connections successfully

- No crashes or failures were observed

- The system remained stable throughout testing

8.  Summary Table

| Devices | CPU Usage | Memory Usage | Network Activity | Status |
| --- | --- | --- | --- | --- |
| 100 | Low | Stable | Moderate | Stable |
| 200 | Low | Stable | Increased | Stable |
| 300 | Low | Stable | High | Stable |
| 400 | Low | Stable | Very High | Stable |
| 500 | Low | Stable | Very High | Stable |

9.  Conclusion

The MQTT broker demonstrated strong scalability and stability under increasing load conditions.

- CPU and memory usage remained consistently low

- Network usage increased proportionally with the number of devices

- The broker successfully handled up to 500 simulated devices without failure

This indicates that the system is capable of supporting a large number of IoT devices efficiently.